**Software Test Plan**

=====================

**Story Key:** AB#26658

**Summary:** MB Block - Normal behavior for PCS Neo

**Description:**

The test plan outlines the requirements for testing the normal behavior of the MB Block in the PCS Neo project.

**Objective:**

To ensure that the MB Block processes binary input signals correctly, producing the expected output signals, internal states, and latching behavior according to configuration.

**Scope:**

* The test plan covers the normal operation and PLC On-Delay and Off-Delay functionality of the MB Block.

* It includes testing with various input scenarios, including binary inputs from hardwired I/O, 3rd party communication, and calculated binary states.

**Prerequisites:**

* PCS Neo project is running

* SE library with MB Block is imported

**Test Scenarios (High Level):**

---------------------------

### Normal Operation

1. Verify correct output signals for input X = 0 and internal BX = 0

2. Verify correct output signals for input X = 1 and internal BX = 1

3. Test latching behavior when P_latch = 1 and X changes from 1 to 0

4. Test de-latching behavior when reset RX changes from 0 to 1

### PLC On-Delay

1. Verify countdown starts and output becomes 1 after delay for input X = 1 and P_OnDelay = 10s

2. Test decimal Off-Delay values are accepted and executed correctly

3. Verify timer runs and output activates after delay for high P_OnDelay values

### PLC Off-Delay

1. Verify output remains ON when input signal is steady state

2. Verify output turns OFF after delay for input X transitions from 1 to 0 and P_OffDelay = 10s

**Test Cases:**

--------------

### Normal Operation

#### Test Case 1:

**Test Title:** Verify correct output signals for input X = 0 and internal BX = 0

**Test Steps:**

* Input X = 0 into the MB Block

* Verify output Y = 0

* Verify internal BX = 0

#### Test Case 2:

**Test Title:** Verify correct output signals for input X = 1 and internal BX = 1

**Test Steps:**

* Input X = 1 into the MB Block

* Verify output Y = 1

* Verify internal BX = 1

#### Test Case 3:

**Test Title:** Test latching behavior when P_latch = 1 and X changes from 1 to 0

**Test Steps:**

* Set P_latch = 1 in the MB Block configuration

* Input X = 1 into the MB Block

* Verify output Y remains ON

* Change input X from 1 to 0

* Verify output Y remains latched at 1

#### Test Case 4:

**Test Title:** Test de-latching behavior when reset RX changes from 0 to 1

**Test Steps:**

* Input X = 1 into the MB Block

* Set reset RX = 0 in the MB Block configuration

* Verify output Y is de-latched to 0

### PLC On-Delay

#### Test Case 5:

**Test Title:** Verify countdown starts and output becomes 1 after delay for input X = 1 and P_OnDelay = 10s

**Test Steps:**

* Set P_OnDelay = 10s in the MB Block configuration

* Input X = 1 into the MB Block

* Verify output Y becomes 1 after delay

#### Test Case 6:

**Test Title:** Test decimal Off-Delay values are accepted and executed correctly

**Test Steps:**

* Set P_OnDelay to a decimal value (e.g., 0.5s) in the MB Block configuration

* Input X = 1 into the MB Block

* Verify output Y responds correctly to the specified delay

#### Test Case 7:

**Test Title:** Verify timer runs and output activates after delay for high P_OnDelay values

**Test Steps:**

* Set P_OnDelay to a high value (e.g., 1200s) in the MB Block configuration

* Input X = 1 into the MB Block

* Verify output Y becomes 1 after specified delay

### PLC Off-Delay

#### Test Case 8:

**Test Title:** Verify output remains ON when input signal is steady state

**Test Steps:**

* Input X = 1 into the MB Block

* Set P_OffDelay to a non-zero value in the MB Block configuration

* Verify output Y remains ON for an extended period

#### Test Case 9:

**Test Title:** Verify output turns OFF after delay for input X transitions from 1 to 0 and P_OffDelay = 10s

**Test Steps:**

* Set P_OffDelay = 10s in the MB Block configuration

* Input X = 1 into the MB Block, then change input X to 0

* Verify output Y turns OFF after specified delay

#### Test Case 10:

**Test Title:** Verify decimal Off-Delay values are accepted and executed correctly

**Test Steps:**

* Set P_OffDelay to a decimal value (e.g., 0.5s) in the MB Block configuration

* Input X transitions from 1 to 0 into the MB Block

* Verify output Y responds correctly to the specified delay

#### Test Case 11:

**Test Title:** Verify high Off-Delay values complete successfully

**Test Steps:**

* Set P_OffDelay to a high value (e.g., 1200s) in the MB Block configuration

* Input X transitions from 1 to 0 into the MB Block

* Verify output Y turns OFF after specified delay